✈

METAR LOOKUP

Version 2.0

USER MANUAL

Real-time Aviation Weather for Windows

Farrand Tech Services

https://www.farrandtech.com

Revised April 2026

Data: aviationweather.gov (NOAA/NWS) • aeronav.faa.gov (FAA)

# Table of Contents

Table of Contents	2

1.  Introduction	4

1.1  System Requirements	4

2.  Getting Started	5

2.1  First Launch	5

3.  The Interface	6

3.1  Top Search Bar	6

3.2  Favorites Strip	6

3.3  Status Indicators	6

4.  Tabs Reference	8

4.1  METAR Tab	8

Header Section	8

Data Tiles	8

ATIS Sections	8

4.2  TAF Tab	9

Header	9

Forecast Period Cards	9

4.3  Map Tab	10

Map Elements	10

Interacting with the Map	10

4.4  Airport Charts Tab	10

Chart List (Left Panel)	10

PDF Viewer (Right Panel)	11

4.5  SIGMETs / AIRMETs Tab	11

Tab Badge	11

Advisory Cards	11

4.6  Calculators Tab	12

Density Altitude Calculator	12

Pressure Altitude Calculator	12

Crosswind / Headwind Component Calculator	12

5.  Dark / Light Theme	14

6.  Settings & Data Persistence	14

7.  API Status Window	14

8.  Troubleshooting	15

8.1  Common Error Messages	15

8.2  Data Appears Stale	15

8.3  Map Does Not Load	15

8.4  Charts Tab Is Empty or Shows an Error	15

9.  Data Sources	16

10.  Quick Reference	16

# 1.  Introduction

METAR Lookup is a free Windows desktop application that gives pilots, student pilots, dispatchers, and aviation enthusiasts instant access to real-time aviation weather data. It consolidates METAR observations, TAF forecasts, SIGMETs/AIRMETs, FAA airport charts, an interactive map, and built-in flight calculators into a single, clean interface.

All data is retrieved live from official sources — primarily the NOAA/NWS Aviation Weather Center (aviationweather.gov) and the FAA AeroNav portal — and is presented using standard aviation conventions (ICAO codes, Zulu time, knots, feet MSL, inHg).

⚠  Safety Notice  METAR Lookup is for informational purposes only. It must not be used as the sole source of weather information for any actual flight operation. Always obtain an official preflight weather briefing.

## 1.1  System Requirements

| Field | Description |
| --- | --- |
| Operating System | Windows 10 version 1809 (build 17763) or later; Windows 11 |
| Architecture | 64-bit (x64) only |
| Internet | Required — all weather data is fetched live from external APIs |
| Display | Minimum 1024 × 768; recommended 1280 × 800 or higher |
| Runtime | .NET 10 Desktop Runtime (bundled in the installer) |

# 2.  Getting Started

## 2.1  First Launch

When the application opens for the first time you will see an empty interface with the search bar at the top. There is no data loaded until you perform your first lookup.

To look up an airport:

- Click the search box labeled “ICAO code (e.g. KSEA).”

- Type the 3- or 4-character ICAO identifier for any airport (e.g. KSEA for Seattle-Tacoma, KORD for Chicago O’Hare, KLAX for Los Angeles).

- Press Enter or click the Lookup button.

- All tabs will populate with live data within a few seconds.

ℹ  ICAO vs IATA codes  METAR Lookup uses ICAO codes, not IATA codes. US airports typically add a ‘K’ prefix: LAX becomes KLAX, ORD becomes KORD, SEA becomes KSEA. Canadian airports start with ‘C’: CYYZ = Toronto. International: EGLL = London Heathrow.

# 3.  The Interface

## 3.1  Top Search Bar

The search bar spans the top of every tab and provides access to the application’s main controls.

| Field | Description |
| --- | --- |
| Search Box | Type a 3–4 character ICAO code and press Enter to look up an airport |
| Lookup Button | Executes the current search (same as pressing Enter in the search box) |
| ☆ Save | Pins the currently displayed airport to the Favorites strip (up to 8) |
| ⚡ API Status | Opens a connectivity check dialog for all external data endpoints |
| 🌙 Dark | Toggles between dark and light theme; preference is saved automatically |

## 3.2  Favorites Strip

The colored chips between the search box and the control buttons are your pinned favorite airports. Each chip is tinted by the flight category that was active when you saved it.

| VFR | Visual Flight Rules — Ceiling ≥ 3000 ft AGL and Visibility ≥ 5 SM |
| --- | --- |
| MVFR | Marginal VFR — Ceiling 1000–3000 ft AGL or Visibility 3–5 SM |
| IFR | Instrument Flight Rules — Ceiling 500–999 ft AGL or Visibility 1–3 SM |
| LIFR | Low IFR — Ceiling < 500 ft AGL or Visibility < 1 SM |

Actions on the Favorites strip:

- Click a chip → instantly looks up that airport in all tabs

- Click × on a chip → removes it from favorites and saves

- Click ☆ Save while viewing an airport → adds it to the strip

- Maximum 8 favorites. Adding a 9th removes the oldest.

## 3.3  Status Indicators

| Field | Description |
| --- | --- |
| Loading bar | A spinning progress ring with a status message appears while data is being fetched |
| Error banner | A red bar below the header shows an error message (invalid code, network failure, etc.) |
| Status bar | The bottom bar shows the last successful lookup time (e.g. “KSEA — updated 14:32 Z”) |

4.  Tabs Reference

## 4.1  METAR Tab

Displays the most recent surface weather observation (METAR) for the selected airport. METARs are typically issued once per hour and updated as special observations when significant changes occur.

### Header Section

| Field | Description |
| --- | --- |
| Flight Category | Color-coded badge (VFR / MVFR / IFR / LIFR) at top-left |
| Airport Name | Full name and city/state (e.g. “Seattle–Tacoma International — Seattle, WA”) |
| Observation Time | UTC time of the observation in HH:mm:ss Z format |
| Observation Date | Calendar date of the observation in YYYY-MM-DD format |
| Raw METAR | The unprocessed METAR string exactly as broadcast, in monospaced font |

### Data Tiles

The grid of tiles shows decoded values from the METAR. Tiles are greyed out when the observation does not include that element.

| Field | Description |
| --- | --- |
| Temperature | Outside air temperature in °C |
| Dewpoint | Dewpoint temperature in °C; higher dewpoint relative to temperature = higher humidity/fog risk |
| Visibility | Prevailing visibility in Statute Miles (SM) |
| Wind Direction | True bearing from which the wind is blowing (e.g. 290°). Displays “VRB” for variable winds |
| Wind Speed / Gust | Sustained wind speed in knots. If gusting, the gust value is shown in orange (e.g. 12 kt G25kt) |
| Altimeter | Altimeter setting in both inHg (US) and hPa (metric/international) |
| Elevation | Airport field elevation in feet MSL and meters |
| Sky Conditions | Cloud layers listed from lowest to highest (e.g. FEW 2500 ft AGL, BKN 5000 ft AGL, OVC). SKC or CLR = clear |

### ATIS Sections

If Automatic Terminal Information Service (ATIS) data is available, arrival and departure broadcasts are shown below the tile grid. These are long-form voice broadcast transcripts that include runway-in-use, approach types, and additional NOTAMs.

## 4.2  TAF Tab

A Terminal Aerodrome Forecast (TAF) is an official weather forecast covering a 24- or 30-hour period for the airspace within 5 nautical miles of an airport. TAFs are issued every 6 hours.

### Header

| Field | Description |
| --- | --- |
| Raw TAF | Full unprocessed TAF text in monospaced font |
| Issued | The date and time the TAF was issued (UTC) |
| Valid Period | The window during which the forecast applies |

### Forecast Period Cards

Each card represents one forecast period within the TAF. Cards are stacked in chronological order.

| Field | Description |
| --- | --- |
| Time Range | UTC window for this period (HH:mm – HH:mm Z) |
| Change Indicator | FM (from), BECMG (becoming), TEMPO (temporary), PROB (probability) |
| Wind | Forecast wind direction and speed with optional gust |
| Visibility | Forecast visibility in Statute Miles |
| Sky Conditions | Forecast cloud layers |
| Category Badge | Color-coded VFR/MVFR/IFR/LIFR badge for the forecast period (top-right of each card) |

ℹ  No TAF available  Smaller or non-towered airports often do not have TAF service. If the tab shows “No TAF available for this station” that is expected behavior, not an error.

## 4.3  Map Tab

An interactive Leaflet.js map shows the searched airport and surrounding METAR stations color-coded by their current flight category.

### Map Elements

| Field | Description |
| --- | --- |
| Primary Marker | Orange glowing pin at the searched airport with its ICAO label |
| Nearby Stations | Colored dots within approximately 60 nautical miles; color matches flight category (green=VFR, blue=MVFR, red=IFR, purple=LIFR) |
| Station Labels | ICAO codes displayed above each marker with a contrasting text shadow for readability |
| Radar Overlay | Semi-transparent precipitation/radar layer from RainViewer (shown when available) |
| Tile Layer | CartoDB Voyager in light mode; CartoDB Dark Matter in dark mode. Switches instantly with the theme toggle. |

### Interacting with the Map

- Pan: click and drag

- Zoom: scroll wheel, or use + / − controls

- Hover over a nearby station marker to see its ICAO code and category

- Click a nearby station marker to open a popup with a “Look Up” link

- Click “Look Up” in the popup to run a full lookup for that station in all tabs

ℹ  Lazy loading  The map only loads when you first click the Map tab. Switching tabs afterward reuses the loaded map without a network request. The radar overlay is fetched separately and may take a moment.

## 4.4  Airport Charts Tab

Provides one-click access to official FAA Terminal Procedures Publication (d-TPP) charts including instrument approach plates, departure procedures, arrival routes, and airport diagrams.

### Chart List (Left Panel)

- Charts are grouped by category (Instrument Approaches, Standard Instrument Departures, STARs, Airport Diagram, etc.)

- Click a category header to expand or collapse its list

- Click any chart name to load it in the right panel

### PDF Viewer (Right Panel)

- The selected chart renders as a full PDF in the embedded browser

- Zoom and pan within the PDF using standard browser controls

- A placeholder message is shown until a chart is selected

ℹ  International airports  FAA d-TPP charts cover US airports only. International airports will show “No FAA charts found.” Use the appropriate ANSP or Jeppesen service for non-US charts.

ℹ  Lazy loading  Charts are only downloaded when you first click the Charts tab for a given airport. The chart index is then cached so subsequent visits are instantaneous.

## 4.5  SIGMETs / AIRMETs Tab

Shows all currently active Significant Meteorological Information (SIGMETs) and Airmen’s Meteorological Information (AIRMETs) for the United States. These advisories are fetched with every airport lookup, regardless of which airport was searched.

### Tab Badge

A red count badge appears on the tab header (e.g. “3”) when active advisories exist. The badge is hidden when there are none.

### Advisory Cards

| Field | Description |
| --- | --- |
| Type Badge | Red label: SIGMET or AIRMET |
| Hazard | Type of hazard and severity (e.g. “TURBULENCE — Severe”, “ICING — Moderate”, “IFR — Mountain Obscuration”) |
| Raw Text | Monospaced reproduction of the original advisory text |
| Validity | From and To times in HH:mmZ format |
| Altitude Block | Vertical extent: lower and upper altitude limits in feet MSL |

⚠  SIGMETs vs AIRMETs  SIGMETs (Significant Meteorological Information) are issued for hazards significant to all aircraft: severe turbulence, severe icing, volcanic ash, tropical cyclones. AIRMETs are for conditions that may affect lighter aircraft or those not equipped for IFR operations.

## 4.6  Calculators Tab

Three standard aviation performance calculators that auto-populate from the current METAR when a new airport is loaded.

### Density Altitude Calculator

Density altitude is pressure altitude corrected for temperature deviation from standard. High density altitude reduces aircraft performance.

| Field | Description |
| --- | --- |
| Pressure Altitude | Input in feet (−2000 to 60000 ft); auto-filled from METAR |
| OAT | Outside Air Temperature in °C (−80 to +60°C); auto-filled from METAR |
| Density Altitude | Calculated result in feet; updates in real time as inputs change |

Formula: DA = Pressure Altitude + 118.8 × (OAT − ISA Temperature)

where ISA Temperature = 15°C − (Pressure Altitude / 1000 × 2°C)

### Pressure Altitude Calculator

Pressure altitude is the altitude indicated when the altimeter is set to 29.92 inHg (standard day). Used as the basis for density altitude and performance charts.

| Field | Description |
| --- | --- |
| Field Elevation | Airport elevation in feet MSL; auto-filled from METAR |
| Altimeter Setting | Current altimeter setting in inHg; auto-filled from METAR (default 29.92) |
| Pressure Altitude | Calculated result in feet; updates in real time |

Formula: PA = Field Elevation + (29.92 − Altimeter Setting) × 1000 ft

### Crosswind / Headwind Component Calculator

Calculates the headwind and crosswind components based on current wind conditions and the active runway.

| Field | Description |
| --- | --- |
| Wind Direction | True wind direction in degrees (0–360); auto-filled from METAR |
| Wind Speed | Wind speed in knots (0–200 kt); auto-filled from METAR |
| Runway Heading | The magnetic heading of the runway in use (e.g. 160 for runway 16L) |
| Headwind | Headwind component in knots (positive = headwind, negative = tailwind) |
| Crosswind | Crosswind component in knots (always positive; direction not shown) |

Formulas:

Headwind = Wind Speed × cos(Wind Direction − Runway Heading)

Crosswind = |Wind Speed × sin(Wind Direction − Runway Heading)|

ℹ  Auto-population  When a new METAR loads, all calculator inputs are automatically filled from the live observation data. You only need to enter the runway heading to get a crosswind component.

# 5.  Dark / Light Theme

The 🌙 Dark / Light toggle button in the top-right corner of the toolbar switches the entire application between a dark and a light visual theme.

- The switch is instant — all tiles, backgrounds, and text update immediately

- The map tile layer also switches: CartoDB Voyager (colorful, light) ↔ CartoDB Dark Matter (muted, dark)

- The map tile swap is performed via JavaScript without reloading the page, so there is no blank screen or flicker

- Your preference is saved automatically and restored the next time the application opens

# 6.  Settings & Data Persistence

METAR Lookup stores a small settings file on your local machine. No data is sent to any server.

| Field | Description |
| --- | --- |
| Favorites | Up to 8 pinned airports stored as ICAO code + last-seen flight category |
| Theme | Dark or light theme preference (boolean) |

# 7.  API Status Window

Clicking the ⚡ API Status button opens a diagnostic dialog that tests connectivity to every external endpoint the application uses. This is useful for diagnosing slow lookups or missing data.

- Each endpoint is tested with a lightweight HEAD or GET request

- A green checkmark indicates a successful response; a red X indicates a timeout or error

- This check does not affect the main application state

ℹ  Network environments  Some corporate networks or strict firewalls may block outbound connections to aviationweather.gov or aeronav.faa.gov. If all APIs show red, check your firewall or proxy settings.

# 8.  Troubleshooting

## 8.1  Common Error Messages

| Field | Description |
| --- | --- |
| “Please enter a valid 3–4 character ICAO code” | The search box was empty or contained fewer than 3 characters. Enter a full ICAO identifier. |
| “Failed to fetch METAR data” | The application could not reach aviationweather.gov. Check your internet connection and try again. |
| “No METAR found for [ICAO]” | The entered ICAO code is not recognised by the weather service. Verify the code is correct. |
| “No TAF available for this station” | Expected for non-towered and small airports that do not have TAF service. |
| “No FAA charts found” | The airport is either international (no FAA d-TPP coverage) or too small to have published instrument procedures. |
| “No active SIGMETs or AIRMETs” | Normal status. No significant meteorological advisories are currently in force. |

## 8.2  Data Appears Stale

METARs are typically updated every hour. If data appears stale, perform a new lookup by pressing Enter in the search box or clicking Lookup. The application always fetches fresh data on each explicit lookup.

## 8.3  Map Does Not Load

- The map requires an internet connection to download tile images from CartoDB.

- If tiles appear grey or the map is blank, check your connection.

- The radar overlay is optional and may not appear if RainViewer is unavailable.

## 8.4  Charts Tab Is Empty or Shows an Error

- Charts are loaded the first time you click the Charts tab.

- If loading fails, switching away from and back to the tab will retry.

- Charts are only available for US airports with published FAA instrument procedures.

# 9.  Data Sources

| Field | Description |
| --- | --- |
| NOAA/NWS Aviation Weather Center | aviationweather.gov — METAR, TAF, SIGMET, AIRMET, nearby station data |
| FAA AeroNav | aeronav.faa.gov — Airport charts (d-TPP Terminal Procedures Publication) |
| airport-data.com | Airport coordinates, elevation, and supplemental metadata |
| RainViewer | rainviewer.com — Radar and precipitation overlay tiles |
| CartoDB | cartocdn.com — Map tile layers (Voyager and Dark Matter themes) |
| Leaflet | leafletjs.com — Open-source interactive map library v1.9.4 |

All external services are accessed over HTTPS. METAR Lookup does not cache weather data between sessions; every lookup retrieves fresh data from the source.

# 10.  Quick Reference

Keyboard and mouse quick reference for METAR Lookup v2.0.

| Action | How | Result |
| --- | --- | --- |
| Look up an airport | Type ICAO → Enter, or click Lookup | All tabs refresh with live data |
| Quick-access favorite | Click any chip in the header strip | Instant lookup for that airport |
| Pin an airport | Click ☆ Save | Airport added to Favorites (max 8) |
| Remove a favorite | Click × on the chip | Removed and saved |
| Toggle theme | Click 🌙 Dark / ☀️ Light | Entire UI + map tile layer updates instantly |
| View a chart | Charts tab → click chart name | PDF renders in right panel |
| Look up a nearby station | Map tab → click marker → Look Up | Full lookup for that station |
| Retry failed lookup | Press Enter in the search box | New fetch from APIs |
| Check API connectivity | Click ⚡ API Status | Diagnostic window opens |

METAR Lookup v2.0  •  User Manual  •  April 2026

Farrand Tech Services

https://www.farrandtech.com/

Data: aviationweather.gov (NOAA/NWS) • Not for operational flight planning